How do I receive text messages and calls when connected using a data-only eSIM?

If your eSIM only supports data, you can still use another number for voice calls and text messages.

Please check your eSIM's "More Information" section to learn about your plan type. You can refer to the following article for instructions on how to do this: Can I use my eSIM to make calls or send text messages?

To use your primary number for voice calls and text messages while enjoying your eSIM's data usage, you need to enable your primary number and set it as your default voice number.

Depending on your local provider's requirements, you may also need to enable data roaming. This may incur roaming charges, so we strongly recommend that you consult your local provider before proceeding.

On iOS: Go to Settings > Cellular

Tap your primary number

Turn on Enable this number

Return to Cellular. Then, tap Default voice number

Select your primary number

Return to the Cellular page. This time, select Cellular

Make sure you are using your eSIM for mobile data.

On Samsung devices:

Option 1: Through the Quick Settings panel

Swipe down from the top of the screen. (You may need to scroll down again to see the full menu)

You should be able to see "Calls | Messages | Mobile Data". Tap Calls.

Select your primary number.

Do the same for Messages.

Make sure you also use your eSIM for mobile data.

Option 2: Through Settings

Go to Settings > Connections > SIM Manager.

Scroll down until you see the Preferred SIM section.

Tap Calls.

Select your primary number.

Do the same for Messages.

Make sure you use your eSIM for mobile data.

On Pixel Devices

(Note that the following steps may vary depending on your Pixel model)

Go to Settings > Network & Internet > SIMS.

Select your primary number.

Switch to using a SIM card and turn it on (if it's not already enabled, proceed to the next step).

Find Call Preferences and tap it.

A window will pop up asking which plan you want to use for calls.

Select your primary number.

Do the same as step 3 for Message Preferences.

For "Mobile Data", make sure eSIM is selected for it.

After completing the above steps, you should set your primary line/main line to call or SMS, and then set your eSIM to mobile data. How can I receive SMS and calls while connected with my data-only eSIM?