How to install an eSIM on a Google Pixel device?

Not all Google Pixel models are eSIM compatible, so it's best to check if your device supports eSIM before you begin. Follow the steps in this article to check if your Pixel device supports eSIM.

Once you've confirmed that your Google Pixel device is eSIM compatible and your carrier has unlocked it, you can follow these steps to set up your eSIM.

What preparations are needed before installing an eSIM?

Sign in to your eSIM account.

Go to My eSIM.

Select the eSIM you want to install.

Open the installation instructions. Depending on your app version, you may see:

View installation instructions or

How to use eSIM > How to install eSIM

Choose your preferred installation method: Direct, QR code, or Manual

For connection setup, open the same eSIM and see How to use eSIM > How to connect (or view installation instructions > Part 2/2).

How to install an eSIM directly on a Pixel device

In the eSIM app, tap Direct > Install eSIM

Select Yes to confirm

Wait for the eSIM installation to complete

Open Settings > Networks & Internet

Select your new eSIM

Turn on Mobile Data & Roaming switches

How to install an eSIM on a Pixel device via QR code

Open Settings > Networks & Internet

Select the + icon next to SIM

Select Do you want to download the SIM card instead?

Select Next, then scan the QR code in the eSIM app

Tap Download to install eSIM

Open Settings again

Turn on Activate, Mobile Data & Roaming switches

How to manually install an eSIM on a Pixel device

Open Settings > Networks & Internet

Select the + icon next to SIM

Select Do you want to download the SIM card instead?

Select Next, then tap Need help?

Select Manual Input

Paste or enter the SM-DP+ address and activation code (and confirmation code, if any) from the eSIM app

Select Continue to install eSIM

Open Settings again

Turn on Activate, Mobile Data & Roaming switches

What happens after installation?

Your eSIM should now appear under Settings > Networks & Internet > SIM and automatically connect to a supported mobile network.

If you need further assistance, please contact our support team. We offer 24/7 service and are happy to help.